# Programming the part with the embedded bootloader

## Checking if the bootloader is ready to use

The first step consists to check if the bootloader is ready to use because it might have been erased before by a full chip erase command. This mainly happens when the JTAGICE mkII device is used previously for programming and debugging operations. It happened also in the past that some EVK’s were delivered without the bootloader code.

The bootloader is started by applying the hardware condition which consists to reset the MCU by means of the reset button while pressing the joystick button down (reset button down Joystick button down - Joystick button up - Reset button up). The board is then in ISP (In-System Programming) mode.

To check if the bootloader is running, launch “System” option from Windows XP/Start/Control Panel menu. Select Hardware tab and cick on device manager. You should get an AT32UCA driver under “LibUSB-Win32 Devices” item.

USB Bootloader’s driver

If the USB bootloader’s driver does not come up in the Device Manager, the bootloader might have been erased and then it advised to flash the bootloader code.

## Flashing the bootloader code

3 operations are necessary to flash and configure the bootloader :

- code programming

- fuses programming

- user’s page programming

Refer to the AVR32 UC3 USB DFU bootloader datasheet to get further information about these operations : 
http://www.atmel.com/dyn/resources/prod_documents/doc7745.pdf

The easiest way to do these operations is to execute the script files provided with the standalone software framework packages which can be downloaded from the web :

- Engineering samples : UC3A-UES

“program_at32uc3aes-isp-1.0.0.sh” supplied in folder \SERVICES\USB\CLASS\DFU\EXAMPLES\ISP\AT32UC3AES\Releases\AT32UC3AES-ISP-1.0.0.

- Industrial samples : UC3A-U

“program_at32uc3a-isp-1.0.2.sh” supplied in folder \SERVICES\USB\CLASS\DFU\EXAMPLES\ISP\AT32UC3A\Releases\AT32UC3A-ISP-1.0.0.

- Engineering samples : UC3B-UES

“program_at32uc3aes-isp-1.0.0.sh” supplied in folder \SERVICES\USB\CLASS\DFU\EXAMPLES\ISP\AT32UC3AES\Releases\AT32UC3AES-ISP-1.0.0.

- Industrial samples : UC3B-U

“program_at32uc3b-isp-1.0.2.sh” supplied in folder \SERVICES\USB\CLASS\DFU\EXAMPLES\ISP\AT32UC3B\Releases\AT32UC3B-ISP-1.0.0.

The above supplied script files runs under Linux environment only. For Windows environment, the script files are provided in folders of the zip file supplied with this document. Replace the folder of your software framework by the according one. Open a DOS window and execute the file with extension .bat.

The script file starts the bootloader automatically at end of  the code flashing operation. No need to apply the hardware condition described in the paragraph “Checking if the bootloader is ready to use” to set the MCU in ISP mode. This means that BatchISP can be run just after if the user’s binary code is available.

## Compiling the user’s code

Before programming the user’s code with BatchISP, it is necessary to compile the user’s code so that it does not overlap the bootloader code.

A file called Trampoline provided with the software framework must be linked with the user’s code sources. Carefully read the usage section of the AVR32 UC3 USB DFU bootloader datasheet to get further explanations.

All code examples provided in the software framework are linked with the Trampoline file so that each code example can be programmed by the JTAGICE mkII device or with the bootloader.

Have a look in one of them to know how to link the Trampoline file with your own code sources.

## Programming the user’s code with BatchISP

Read “Batchisp Support Package” section available from AVR32 studio’s help content menu to have detailed explanations how to configure and use BatchISP to program the application into the MCU in ISP mode.